Dual-Method Spinal Curvature Analysis

with AI Calibration Layer

Complete Research & Implementation Notebook

Pipelines Included:

Anatomical Method (SpinePose CVPR 2025)  |  Geometric Method (MediaPipe)  |  AI Calibration (RandomForest + MLP)

RESEARCH DISCLAIMER: All measurements are derived from 2D projections and are sensitive to camera alignment, perspective distortion, and subject orientation. The method assumes sagittal-plane motion and does not account for 3D spinal rotation.

Datasets: Mendeley Posture Dataset  |  SpineTrack  |  Own Collected Dataset

# 1. Project Overview & Research Goal

FINAL RESEARCH GOAL: Build a dual-method spinal analysis system + AI calibration layer that (1) compares geometric vs anatomical curvature, (2) explains their differences scientifically, and (3) reduces error using learning-based correction. This constitutes the publishable contribution.

This project implements a complete pipeline for non-invasive spinal posture analysis from video. Two complementary methods are combined:

| Component | Method | Technology | Purpose |
| --- | --- | --- | --- |
| Anatomical Pipeline | Cobb Angle | SpinePose CVPR 2025 (37 keypoints) | Gold-standard reference |
| Geometric Pipeline | Surface Curvature | MediaPipe + Polynomial Fitting | Video-accessible proxy |
| AI Calibration | Regression Model | RandomForest + PyTorch MLP | Error reconciliation |
| Validation | Clinical Ranges | Mendeley Posture Dataset | Biomechanical grounding |

## 1.1 System Architecture

The pipeline follows five stages:

- Video Input → Frame extraction at native FPS

- Dual Detection → SpinePose keypoints + MediaPipe silhouette in parallel

- Angle Computation → Robust Cobb (FIX-1 + FIX-2) and inclination-difference

- Temporal Smoothing → Savitzky-Golay filter (FIX-3 + FIX-4)

- AI Calibration → Feature matrix → RandomForest/MLP correction

## 1.2 Key Fixes Applied

All five recommended fixes from the research review have been implemented:

| Fix # | Problem | Solution | Impact |
| --- | --- | --- | --- |
| FIX-1 | Raw max-min Cobb is outlier-sensitive | Top-2 / Bottom-2 mean averaging | Stabilizes Cobb estimation |
| FIX-2 | Forward lean conflated with curvature | Subtract global trunk lean bias | Separates lean from kyphosis |
| FIX-3 | Noisy temporal signal | Savitzky-Golay + moving average | Smooth, publication-ready curves |
| FIX-4 | Extreme edge biased by clothing | Centerline (left+right)/2 contour | Reduces silhouette bias |
| FIX-5 | No frame quality gating | Strict confidence + ordering check | Only valid frames analysed |

# 2. Dataset Strategy

Dataset philosophy: Use existing datasets for validation structure, and your own collected videos as the primary training + evaluation set. This is the realistic approach given the absence of radiographic ground truth.

## 2.1 Mendeley Posture Dataset — Clinical Validation

Purpose: Defines normal/mild/severe Cobb angle ranges from literature. Used to validate SpinePose output against clinically accepted thresholds. Not used for training.

| Condition | Kyphosis Range (°) | Geometric Range (°) | Clinical Source |
| --- | --- | --- | --- |
| Normal | 20 – 40 | 0 – 20 | Mendeley / Literature |
| Mild | 40 – 60 | 20 – 40 | Mendeley / Literature |
| Severe | > 60 | > 40 | Mendeley / Literature |

Integration: config.py loads these as CLINICAL_RANGES. calibration_model.py's validate_against_clinical_ranges() checks all predictions against these thresholds and reports the proportion of frames in each clinical category.

## 2.2 SpineTrack Dataset — Pipeline Structure Testing

Purpose: Provides annotated keypoint data for spinal landmarks. Used to verify the SpinePose detection chain and to test the anatomical pipeline's keypoint ordering logic. Not used for calibration training.

- Splits available: train / val / test (CSV format)

- Loader: calibration_model.load_spinetrack_keypoints(split='train')

- Primary use: Detect systematic keypoint drift or ordering violations

## 2.3 Own Collected Dataset — Training & Evaluation (MOST IMPORTANT)

Purpose: Primary training and evaluation dataset for the calibration model. Collected in controlled sagittal-view conditions.

| Property | Target Specification |
| --- | --- |
| Number of videos | 50 – 200 (side-view, sagittal plane) |
| Labels | normal / mild / severe (manual annotation) |
| Camera setup | Fixed tripod, side view, neutral background |
| Subject distance | 2 – 4 m from camera |
| Label file | own_dataset/labels.csv (filename, label columns) |
| Feature CSV | own_dataset/features.csv (auto-generated by pipeline) |

Recording protocol: Each subject performs 30 seconds of natural standing, 30 seconds of exaggerated forward lean, and 30 seconds of normal gait. This gives temporal variation across all three severity levels.

# 3. Anatomical Pipeline (spine_analysis.py)

## 3.1 SpinePose 37-Keypoint Model

SpinePose (CVPR 2025) predicts 37 anatomical landmarks for the full spine. The system uses 9 vertebral landmarks from C1 to Sacrum:

| Landmark | Keypoint Index | Spinal Region | Role in Pipeline |
| --- | --- | --- | --- |
| C1 | 36 | Cervical | Apex of cervical curve |
| C4 | 35 | Cervical | Mid-cervical reference |
| C7 | 18 | Cervicothoracic junction | Cobb angle top |
| T3 | 30 | Thoracic | Upper thoracic segment |
| T8 | 29 | Thoracic | Mid-thoracic reference |
| L1 | 28 | Thoracolumbar junction | Cobb angle bottom |
| L3 | 27 | Lumbar | Mid-lumbar reference |
| L5 | 26 | Lumbosacral | Lower lumbar reference |
| Sacrum | 19 | Sacral | Trunk lean baseline |

## 3.2 Robust Cobb Angle Computation (FIX-1 + FIX-2)

The Cobb angle is computed between vertebral segments C7–T3, T3–T8, and T8–L1. Each segment's inclination is calculated as:

inclination = atan2(horizontal_displacement, |vertical_displacement|)   [degrees]

FIX-1: Instead of taking raw max − min (which is outlier-sensitive), the robust estimator uses:

top    = mean(highest 2 inclinations)

bottom = mean(lowest 2 inclinations)

cobb   = |top - bottom|

FIX-2: The global trunk lean (C7 → Sacrum inclination) is subtracted from all segment inclinations before computing cobb, so the result reflects pure curvature rather than forward tilt:

lean           = segment_inclination(C7, Sacrum)

normalized[i]  = inclination[i] - lean

cobb           = robust_difference(normalized)

## 3.3 Frame Validity (FIX-5)

Each frame is accepted only if:

- Mean keypoint confidence across all 9 spine landmarks >= 0.5 (configurable in config.SPINEPOSE_CONFIG)

- Anatomical ordering is preserved: C7.y < T3.y < T8.y < L1.y < Sacrum.y (top-to-bottom in image)

Rejected frames are categorized as: low_confidence | ordering_violation | no_detection | no_estimation | error

## 3.4 Temporal Smoothing (FIX-3)

Savitzky-Golay filter (window=9, polynomial degree=2) is applied to kyphosis and trunk lean time series. If the window is too short for the filter, it falls back to a symmetric moving average.

smoothed = savgol_filter(arr, window=9, polyorder=2, mode='interp')

# 4. Geometric Pipeline (surface_curvature_analysis.py)

IMPORTANT: Geometric curvature is NOT anatomically equivalent to Cobb angle. It approximates back-surface shape and is calibrated against SpinePose output. It is labelled as 'geometric curvature' throughout and is never directly compared to clinical Cobb thresholds without calibration.

## 4.1 Segmentation

Person silhouette extracted using MediaPipe Selfie Segmentation (model_selection=1, which is the full-body model optimised for landscape images). Conditions for acceptance:

- Segmentation mask area >= 500 pixels

- At least one connected component present

- Largest contour selected as the person region

If MediaPipe returns a low-quality mask (area < threshold), a GrabCut-inspired luminance thresholding fallback is applied automatically.

## 4.2 Centerline Contour Extraction (FIX-4)

BEFORE (buggy): Only the extreme left or right edge of the silhouette was used — this introduced systematic bias from clothing, arms, and camera angle.

# BEFORE (biased)

bx = cs[0]  if use_left_edge  else cs[-1]

# AFTER (FIX-4): centerline

cx = int((cs[0] + cs[-1]) / 2)    # center of silhouette

The centerline approximates the spine's lateral position and is substantially more stable against clothing and arm interference. IQR outlier rejection (1.5 × IQR) is then applied to the x-coordinate series.

## 4.3 Polynomial Curve Fitting

A cubic polynomial (degree=3) is fitted to the centerline contour between shoulder level (y_top = y0 + 18% of person height) and hip level (y_bot = y0 + 65% of person height). Savitzky-Golay pre-smoothing is applied to the x-coordinates before fitting to reduce noise sensitivity.

poly = np.poly1d(np.polyfit(y_vals, x_smoothed, deg=3))

## 4.4 Angle Computation

The standardized inclination angle is computed from the first derivative of the fitted polynomial at the top and bottom of the region:

angle_top = degrees(atan2(dp(y_top),  1.0))

angle_bot = degrees(atan2(dp(y_bot),  1.0))

curvature = |angle_top - angle_bot|

This convention matches the anatomical pipeline's segment_inclination formula, making the two methods directly comparable.

# 5. AI Calibration Layer (calibration_model.py)

KEY CONTRIBUTION: A learning-based calibration layer is introduced to reconcile discrepancies between geometric surface curvature and anatomical keypoint-based estimates. This is the primary scientific contribution of the project.

## 5.1 Feature Engineering

Six features are computed per frame from both pipelines:

| Feature | Source | Meaning |
| --- | --- | --- |
| spinepose_angle | Anatomical pipeline | Smoothed Cobb angle from SpinePose |
| surface_curvature | Geometric pipeline | Smoothed geometric curvature angle |
| angle_diff | Both | spinepose_angle − surface_curvature (disagreement) |
| trunk_lean | Anatomical pipeline | Global lean bias removed (FIX-2) |
| confidence | SpinePose | Mean keypoint confidence for the frame |
| curvature_variance | Geometric pipeline | Rolling variance over 5 frames (stability) |

The target variable is spinepose_angle_smoothed — the anatomical estimate is treated as the gold-standard reference because it is grounded in anatomical landmarks.

## 5.2 RandomForest Baseline Model

The primary calibrator is a RandomForestRegressor trained on the combined feature matrix from all videos:

RandomForestRegressor(n_estimators=200, random_state=42, n_jobs=-1)

Training procedure:

- 80/20 train-test split (stratified by video label when possible)

- StandardScaler normalization applied to feature matrix

- Feature importance extracted and reported for interpretability

- Model saved via joblib for later deployment

## 5.3 MLP Deep Calibration Model (PyTorch)

An optional small neural network provides a stronger baseline for larger datasets:

| Layer | Output Size | Activation | Notes |
| --- | --- | --- | --- |
| Input | 6 | — | 6 engineered features |
| Linear | 64 | ReLU | — |
| Dropout | 64 | — | p = 0.2 (regularization) |
| Linear | 32 | ReLU | — |
| Output | 1 | — | Corrected angle (°) |

Training: Adam optimizer, MSE loss, 100 epochs, batch size 32. Use MLPCalibrator() in place of RandomForestCalibrator() for larger (> 5,000 frame) datasets.

## 5.4 Expected Performance Comparison

Based on the roadmap's error reduction trajectory:

| Method | Expected MAE | STD Residuals | Notes |
| --- | --- | --- | --- |
| Geometric only | High (~10-15°) | High | No calibration |
| SpinePose only | Medium (~5-8°) | Medium | Depends on keypoint quality |
| AI Corrected (RF) | Low (~2-4°) | Low | Primary contribution |
| AI Corrected (MLP) | Lower (if large dataset) | Lowest | Requires 5k+ frames |

# 6. Unified Pipeline (unified_pipeline.py)

## 6.1 Running the System

Three execution modes:

python unified_pipeline.py                  # Full run (both pipelines + calibration)

python unified_pipeline.py --build-dataset  # Build features.csv from own dataset

python unified_pipeline.py --eval-only      # Load saved model, evaluate only

## 6.2 Output Structure

| Directory | Contents |
| --- | --- |
| outputs/figures/ | PNG comparison plots: one per video + overview bar chart |
| outputs/csv/ | Per-frame results for both pipelines (geometric + anatomical) |
| outputs/report/ | summary_statistics.csv with mean/std/confidence per video |
| models/calibration/ | rf_calibrator.joblib (RandomForest) or mlp_calibrator.pt |
| data/datasets/own/ | features.csv (auto-generated feature matrix for all videos) |

## 6.3 config.py Reference

All parameters are centralized in config.py. Key settings to tune:

| Parameter | Default | Meaning |
| --- | --- | --- |
| SPINEPOSE_CONFIG['conf_thresh'] | 0.5 | Min keypoint confidence to accept frame |
| CONTOUR_CONFIG['mask_area_min'] | 500 | Min silhouette pixels for geometric method |
| CONTOUR_CONFIG['smoothing_window'] | 9 | Savitzky-Golay / moving average window |
| CONTOUR_CONFIG['poly_degree'] | 3 | Polynomial degree for curve fitting |
| CALIBRATION_CONFIG['n_estimators'] | 200 | RandomForest trees |
| CALIBRATION_CONFIG['test_size'] | 0.2 | Holdout fraction for calibration evaluation |
| REJECTION_RATE_WARN | 40.0 | Print warning if rejection rate > this % |

# 7. Evaluation & Validation

## 7.1 Quantitative Metrics

All three methods are evaluated with the same metrics for fair comparison:

| Metric | Formula | Interpretation |
| --- | --- | --- |
| MAE | mean(¦y_pred - y_true¦) | Lower = better; < 5° is clinically acceptable |
| STD Residuals | std(y_pred - y_true) | Spread of errors; lower = more consistent |
| Correlation R | Pearson corr(y_pred, y_true) | Closer to 1.0 = better tracking |
| Rejection Rate | rejected / total × 100 | Higher = less data; < 40% recommended |
| Smoothness | std(diff(signal)) | Lower = less jitter in time series |

## 7.2 Clinical Validation (Mendeley Ranges)

Each method's predictions are cross-checked against the Mendeley-derived clinical ranges. The pipeline reports the proportion of frames classified as Normal / Mild / Severe for each method, allowing comparison of clinical agreement.

## 7.3 Methodology Section (for Paper)

KEY SENTENCE: A learning-based calibration layer is introduced to reconcile discrepancies between geometric surface curvature and anatomical keypoint-based estimates.

Recommended paper methodology structure:

- Section 3.1 — Dual pipeline description (Anatomical + Geometric)

- Section 3.2 — Standardized angle computation (inclination-based)

- Section 3.3 — Preprocessing: strict frame filtering + temporal smoothing

- Section 3.4 — Feature extraction (6 cross-pipeline features)

- Section 3.5 — Calibration model training and evaluation

- Section 3.6 — Validation against Mendeley clinical ranges

# 8. Execution Roadmap

| Week | Tasks | Deliverable |
| --- | --- | --- |
| Week 1 | Deploy fixed pipelines. Verify on existing videos. Check rejection rates. | Clean baseline results + rejection logs |
| Week 2 | Collect 50–200 own dataset videos (side view). Label normal/mild/severe. | own_dataset/labels.csv + videos |
| Week 3 | Run unified_pipeline.py --build-dataset. Train RandomForest calibrator. | rf_calibrator.joblib + features.csv |
| Week 4 | Evaluate all three methods. Compute MAE, STD, R. Validate vs Mendeley. | summary_statistics.csv + figures |
| Week 5 | Write paper methodology + results sections. Generate publication figures. | Draft paper + final plots |

## 8.1 Quick Start Checklist

- [ ] Install SpinePose from CVPR 2025 repo and set model_path in config.py

- [ ] Place video files in data/raw_videos/ with names matching config.VIDEO_INFO

- [ ] Download Mendeley Posture Dataset → data/datasets/mendeley_posture/

- [ ] Download SpineTrack → data/datasets/spinetrack/

- [ ] Record own dataset → data/datasets/own_dataset/ + labels.csv

- [ ] Run: python unified_pipeline.py

- [ ] Check outputs/report/summary_statistics.csv for results

# 9. Source File Reference

| File | Purpose | Key Functions |
| --- | --- | --- |
| config.py | Central configuration | All paths, thresholds, model params |
| spine_analysis.py | Anatomical pipeline | robust_multi_segment_cobb(), is_valid_frame(), analyze_video_anatomical() |
| surface_curvature_analysis.py | Geometric pipeline | get_back_contour_centerline(), compute_inclination_angle(), analyze_geometric_curvature() |
| calibration_model.py | AI calibration + dataset utils | RandomForestCalibrator, MLPCalibrator, build_feature_matrix(), load_mendeley_ranges() |
| unified_pipeline.py | Master orchestration | main(), run_calibration(), plot_video_comparison(), write_summary_report() |

FINAL NOTE: This project is strong engineering. With the five fixes applied, the calibration layer trained, and the Mendeley validation integrated, it becomes publishable research. The key contribution is not just the dual pipeline — it is the AI calibration that learns when each method is reliable and corrects for their systematic differences.

Dual-Method Spinal Curvature Analysis — Complete Research Notebook